PROFORMA INVOICE

Quotation no: B&F PI-302

Date 08/11/2024

To,

B&F Realty-Villa 3

Goa

Dear Sir,

Kindly find the quotation as per your requirement.

ENDO

| PROJECT : RIVER GLADE VILLAS; BILL OF QUANTITIES FOR RIVER GLADE VILLAS - Villa 03; Date: 24 SEPT 2024. Notes : Included are luminaires suggested for - Interior Lighting for VILLA-03 Kindly Refer the detail specifications with this BOQ. If you Find any descripancies , please get it to our notice before placing final order.. |  |  |
| --- | --- | --- |
| Sr No | Specification Description |  |  |  |  | Symbol |  | Manufacturer |  |  | Total Quantity |  | Remarks |  |  |  |
| 1 | LED Strip Light with flat Channel & Clear Cover. |  |  |  |  |  |  | OSRAM LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Extruded aluminium Channel & clear Cover. |  |  |  |  | COV1 |  | LFP 1200S-G5-930-10 L1 |  |  | 57.50 |  | The actual Lengths required to be confirmed as per site. |  |  |  |
|  | Dimension: 17.1 X 8mm |  |  |  |  |  |  |  |  |  | Mtrs. |  |  |  |  |  |
|  | Accessory: DG-A002 with Clear Cover |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lamp: 11.6W/Mt, 3000K, CRI=90+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output :1100lm/Mt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 11.6W/Mt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 | LED Strip Light along circular cove. |  |  |  |  |  |  | OSRAM LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: 24 V LED strips flexible and cuttable, 50 mm cut unit, with superior optically graded silicone encapsulation. |  |  |  |  | COV1c |  | GFDS600-G1-830-01 |  |  | 14.00 |  | The actual Length required in Circular cove, to be confirmed as per Site. |  |  |  |
|  | Dimension: 10 X 10mm |  |  |  |  |  |  |  |  |  | Mtrs. |  |  |  |  |  |
|  | Accessory: Fixing clips |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lamp: 9W/Mt LED, 110º 3000K, CRI=90+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 620lm/Mt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 9W/Mt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | LED Strip light with 45° Channel & Diffuse Cover. |  |  |  |  |  |  | OSRAM LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Extruded aluminium Channel & 45° Cover |  |  |  |  | COV2 |  | LFP800S -G5-930-10 L1 |  |  | 104.50 |  | Mounted in wardrobes & Curtain pelmets. Actual lengths to be confirmed as per site. |  |  |  |
|  | Dimension: 16 X 16mm |  |  |  |  |  |  |  |  |  | Mtrs. |  |  |  |  |  |
|  | Accessory: DG-A008 with Diffuse Cover |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lamp: 8W/Mt LED, 3000K, CRI=90+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 840 lm/Mt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 8W/Mt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | LED Strip Light with Flat Channel & Dotless Diffuse Cover |  |  |  |  |  |  | OSRAM LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Extruded aluminium Channel & Diffuse Cover |  |  |  |  | COV3 |  | LFP800S -G5-930-10 L1 |  |  | 67.50 |  | For task Lighting in Kitchen. Actual lengths to be confirmed as per Site. |  |  |  |
|  | Dimension:17.1mm X 8mm |  |  |  |  |  |  |  |  |  | Mtrs. |  |  |  |  |  |
|  | Accessory: DG-A002 with Diffuse Cover |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lamp: 8W/Mt LED, 3000K, CRI=90+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output :840lm/Mt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 8W/Mt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 | LED Strip Light with Channel & Dotless Diffuse Cover |  |  |  |  |  |  | OSRAM LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Extruded aluminium Channel & Diffuse Cover |  |  |  |  | COV4 |  | LF500-G1-930-10 L2 |  |  | 128.50 |  | Actual Length to be confirmed as per site. |  |  |  |
|  | Dimension: 17.1mm X 8mm |  |  |  |  |  |  |  |  |  | Mtrs. |  |  |  |  |  |
|  | Accessory: DG-A010 with Diffuse Cover |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lamp: 3.9W/Mt LED, 3000K, CRI=90+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 400lm/Mt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 3.9W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 | Flex Light with Top View |  |  |  |  |  |  | OSRAM LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Silicone |  |  |  |  | FLX1 |  | GFDT400-G1-830-01 |  |  | 9.20 |  | Actual Length to be confirmed as per site. |  |  |  |
|  | Dimension: 10mm X 10mm |  |  |  |  |  |  |  |  |  | Mtrs. |  |  |  |  |  |
|  | Accessory: Mounting Clips. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lamp: 6W/Mts LED ,110° CRI=90+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 420lm/Mts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 6W/Mts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 | Ceiling Recessed LED Down Light (IP Rated) |  |  |  |  |  |  | ENDO LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Die cast Aluminium |  |  |  |  | IPR1 |  | GERD8901W-T |  |  | 33.00 |  |  |  |  |  |
|  | Dimension:Trim∅=60mm, Ht=95mm |  |  |  |  | White Trim |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp:10.2W LED, 22° Fixed,3000K, CRI=90+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 446lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 10.2W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP44 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | In-ground LED Up lighter. |  |  |  |  |  |  | PUK LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Aluminium Body & Stainless Steel Trim |  |  |  |  | IUL1 |  | 201073 |  |  | 15.00 |  |  |  |  |  |
|  | Dimension: Trim∅=81mm, & Ht=85mm |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 5W LED, 10°, 3000K CRI=80+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 489lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Non-Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 5W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | Profile System , Recessed Magnetic Track. |  |  |  |  |  |  | DELTA LIGHTING |  |  |  |  | Drawing to be shared with Vendor to finalize unit modules & connectors. |  |  |  |
|  | Housing: Anodised Aluminum |  |  |  |  | PSY1 |  | Splitline M20 |  |  | 12.00 |  |  |  |  |  |
|  | Dimensionin Section : W=53mm, D=50mm |  |  |  |  |  |  |  |  |  | Mtrs. |  |  |  |  |  |
|  | Accessory: Connector Sets - As per Vendor. |  |  |  |  |  |  |  |  |  | LEngth |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9a | Recessed Linear LED light in Profile System. |  |  |  |  |  |  | DELTA LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Anodised Aluminum |  |  |  |  | M1 |  | LND-06-SP-30F-B-M-MDL (23303 9000) |  |  | 8.00 |  |  |  |  |  |
|  | Dimension: L=621mm W=20mm, D=50mm |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Accessory: SplitLine M20 - Profile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lamp: 9.1W/621mm LED, 3000K, CRI=90+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 625lm/621mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 9.1W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9b | Recessed LED Twin Spot light in Profile System. |  |  |  |  |  |  | DELTA LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Anodised Aluminum |  |  |  |  | M2 |  | B (233179320) |  |  | 12.00 |  |  |  |  |  |
|  | Dimension: TrimØ=39mm, L=170mm H=116mm |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Accessory: SplitLine M20 - Profile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lamp: 2X8.5W LED, 3000K, CRI=90+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 1089lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 2X8.5W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Ceiling Recessed LED Pin Spot Down Light. |  |  |  |  |  |  | ENDO LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Die cast aluminium |  |  |  |  | PNS1 |  | GERD9898W-T |  |  | 5.00 |  |  |  |  |  |
|  | Dimension: Trim∅=37mm, CutOut∅=35mm Ht=42mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 3.1W LED 29°, Focusable, 3000K, CRI=93+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output: 178lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Cosumption: 3.1W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Ceiling Recessed LED Pin Spot Down Light. |  |  |  |  |  |  | ENDO LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Die cast aluminium |  |  |  |  | PNS2 |  | GERD9869W-T |  |  | 8.00 |  |  |  |  |  |
|  | Dimension: Trim∅=60mm,Cut Out∅=50mm Ht=101mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 10.3W LED 21°, Focusable, 3000K, CRI=93+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output: 446lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Cosumption: 10.3W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 | Ceiling Recessed LED Down Light. |  |  |  |  |  |  | ENDO LIGHTING |  |  |  |  |  |  |  |  |
|  | Housing: Die cast aluminium |  |  |  |  | RDN1 |  | GERD8280WB-T |  |  | 37.00 |  |  |  |  |  |
|  | Dimension:∅=85mm, H=135mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 19.6W LED 20°, Focusable, 3000K, CRI=93+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output: 807lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Cosumption: 19.8W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Ceiling Reccesed LED Down Light. |  |  |  |  |  |  | ENDO Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Die cast aluminium |  |  |  |  | RDN2 |  | GERD8282WB-T |  |  | 23.00 |  |  |  |  |  |
|  | Dimension:L=∅=85mm, H=135mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 19.6W LED 33°, Focusable, 3000K, CRI=93+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 1116lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 19.8W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Ceiling Reccesed LED Twin Down Light.(Black Trim Required) |  |  |  |  |  |  | ENDO Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Die cast aluminium |  |  |  |  | RDN3 |  | GERD8849W-T |  |  | 32.00 |  | Black Trim Finish (I.D to confirm) |  |  |  |
|  | Dimension: o 122 X65mm & H=115mm |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp:14.2W LED, 26°, Focusable, 3000K, CRI=93+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 1254lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 14.2W/Unit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | Ceiling Recessed LED Down Light |  |  |  |  |  |  | ENDO Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Die cast aluminium |  |  |  |  | RDN4b |  | GERD8127WA-T |  |  | 15.00 |  | Black Trim Finish (In Bedroom 04 sloping ceiling) |  |  |  |
|  | Dimension: ∅=108mm, Depth=100mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 19.8W LED,19°, Focusable ,3000K CRI=96+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 1377lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 19.8W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | Ceiling Recessed LED Down Light |  |  |  |  |  |  | ENDO Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Die cast aluminium |  |  |  |  | RDN4w |  | GERD8127WA-T |  |  | 12.00 |  | White Trim Finish (In Terrace Lobby sloping ceiling) |  |  |  |
|  | Dimension: ∅=108mm, Depth=100mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 19.8W LED,19° , Focusable ,3000K CRI=96+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 1377lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 19.8W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Ceiling Recessed LED Down Light |  |  |  |  |  |  | ENDO Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Die cast aluminium |  |  |  |  | RDN5 |  | GERD8202WB-T |  |  | 5.00 |  |  |  |  |  |
|  | Dimension: ∅=108mm, Depth=133mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 28.5W LED, 32°, Fixed Optics, 3000K CRI=83+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 1597lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 28.5W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Surface Mounted LED Down Light. |  |  |  |  |  |  | ABBY Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Die cast aluminium |  |  |  |  | SDN1 |  | iPlane round surface-15W |  |  | 14.00 |  |  |  |  |  |
|  | Dimension: ∅=152mm & H=38mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 15W LED, 116°, Fixed, 3000K, CRI=96+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 1209lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Integral LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 15W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP44 Required |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Ceiling Recessed LED Light, Fixed. |  |  |  |  |  |  | ABBY Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Spun Aluminium |  |  |  |  | SDN2 |  | SPA 63 - 22W |  |  | 12.00 |  |  |  |  |  |
|  | Dimension: ∅=94mm, Cutout∅=84 & Height=104mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 22W LED, 32° Fixed, 3000K, CRI=90+(R9≥40+) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 1704 lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 22W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | minimum IP44 Required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Spike Mounted LED Projector Light. |  |  |  |  |  |  | PUK Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Hard Anodized Aluminium |  |  |  |  | SPK1 |  | 101903 |  |  | 11.00 |  |  |  |  |  |
|  | Accessories : Anti-glare snoot (AC098)& Spike (AC044) |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Dimension: ∅=50mm, Ht=90mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lamp: 10W LED,10º, 3000K CRI=80+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 489lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Non-Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 10W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 | Spike Mounted LED Projector Light. |  |  |  |  |  |  | PUK Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Hard Anodized Aluminium |  |  |  |  | SPK2 |  | 101901 |  |  | 11.00 |  |  |  |  |  |
|  | Accessory: Anti-glare snoot (AC098) & Ground Burried Spike (AC044) |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Dimension: ∅=35mm, Ht=63mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lamp: 3W LED, 45º, 3000K CRI=80+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 230lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Non-Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 3W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 | Underwater LED Up Lighter. |  |  |  |  |  |  | PUK Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Stainless Steel |  |  |  |  | UWL3 |  | 301004 |  |  | 2.00 |  | Black Finish Required |  |  |  |
|  | Dimension: TrimØ=81mm, Cut OutØ=79mm & Ht=100mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 6W LED, 10°, 3000K, CRI=80+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 576lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 6W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 | Wall Mounted LED Light. |  |  |  |  |  |  | PUK Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Die cast Aluminium |  |  |  |  | WML1 |  | 402403 |  |  | 5.00 |  |  |  |  |  |
|  | Dimension: L=354mm, W=120mm & Ht=36mm. |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 12W LED, 3000K, CRI=80+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 574lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Integral LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 12W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | Wall Recessed LED Light. |  |  |  |  |  |  | PUK Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Die cast Aluminium |  |  |  |  | WRL1 |  | 502017 |  |  | 11.00 |  |  |  |  |  |
|  | Dimension: Trim Size=255 X 76mm, Depth= 81mm |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 7W LED, 120°, 3000K, CRI=80+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 180lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Integral LED Driver. Non- Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 7W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 | Wall Recessed LED FootLight. |  |  |  |  |  |  | PUK Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Stainless Steel |  |  |  |  | WRL2 |  | 502015 |  |  | 21.00 |  |  |  |  |  |
|  | Dimension: Trim Size=79 X 79mm, Depth= 77mm |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 1W LED, 3000K, CRI=80+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 60lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Integral LED Driver. Non- Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 1W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | Wall Recessed LED Light (To be Customize to 4W). |  |  |  |  |  |  | PUK Lighting |  |  |  |  |  |  |  |  |
|  | Housing: Die cast Aluminium |  |  |  |  | WRL3 |  | 502017 |  |  | 6.00 |  |  |  |  |  |
|  | Dimension: Trim Size=255 X 76mm, Depth= 81mm |  |  |  |  |  |  |  |  |  | Nos |  |  |  |  |  |
|  | Lamp: 4W LED, 120°, 3000K, CRI=80+ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lumen Output : 100lm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Control Gear: Integral LED Driver. Non- Dimmable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Power Consumption: 4W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IP65 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| DATE: 07 NOV 2024 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ENDO ALTERNATES |  |  |  |  |  |  |  |
|  |  | Sr No | Specification Description | Symbol | Manufacturer | Total Quantity | Remarks/ L@W Comments | BEST PRICE | Total |
|  |  | 1 | LED Strip Light with flat Channel & Clear Cover. |  | ENDO LIGHTING |  |  |  |  |
|  |  |  | Housing: Extruded aluminium Channel & clear Cover. | COV1 | TSUKI-R-1714-10W/Mtr | 57.50 | SUGGEST IN 10W/Mt. The actual Lengths required to be confirmed as per site., OLD QTY 55 MTR | 1172 | 67390 |
|  |  |  | Dimension: 17 X 14mm |  |  | Mtrs. |  |  |  |
|  |  |  | Accessory: Channel & Clear Cover. |  |  |  |  |  |  |
|  |  |  | Lamp: 15W/Mt, 3000K, CRI=90+ |  |  |  |  |  |  |
|  |  |  | Lumen Output :1500lm/Mt |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 10W/Mt |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  |  |  |  | 80W, 24V Dali Driver | 12.00 |  | 4992 | 59904 |
|  |  | 2 | LED Strip Light along circular cove. |  | ENDO LIGHTING |  |  | 4425 | 61950 |
|  |  |  | Housing: Silicone | COV1c | TSUKI-NF-DB-16X16-NS-130- 14W-IP67 | 14.00 | OK. The actual Length required in Circular cove, to be confirmed as per Site. |  |  |
|  |  |  | Dimension: 16 X 16mm |  |  | Mtrs. |  |  |  |
|  |  |  | Accessory: Fixing Clips |  |  |  |  |  |  |
|  |  |  | Lamp: 14W/Mt LED, 120° 3000K, CRI=85+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : 710lm/Mt |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 14W/Mt |  |  |  |  |  |  |
|  |  |  | IP67 |  |  |  |  |  |  |
|  |  |  |  |  | 80W, 24V Dali Driver | 3.00 |  | 4992 | 14976 |
|  |  |  |  |  |  | Nos |  |  |  |
|  |  | 3 | LED Strip light with 45° Channel & Diffuse Cover. |  | ENDO LIGHTING |  |  | 1319 | 137836 |
|  |  |  | Housing: Extruded aluminium 45° Channel & Diffuse Cover | COV2 | TSUKI-R-1818-10W/Mtr | 104.50 | OK. Mounted in wardrobes & Curtain pelmets. Actual lengths to be confirmed as per site. OLD QTY 88 MTR |  |  |
|  |  |  | Dimension: 18 X18mm |  |  | Mtrs. |  |  |  |
|  |  |  | Accessory: 45° Channel with Diffuse Cover |  |  |  |  |  |  |
|  |  |  | Lamp: 10W/Mt LED, 3000K, CRI=90+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : 1000lm/Mt |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 10W/Mt |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  |  |  |  | 80W, 24V Dali Driver | 21.00 |  | 4992 | 104832 |
|  |  | 4 | LED Strip Light with Flat Channel & Dotless Diffuse Cover |  | ENDO LIGHTING |  |  | 1172 | 79110 |
|  |  |  | Housing: Extruded aluminium Channel & Diffuse Cover | COV3 | TSUKI-R-1714-10W/Mtr | 67.50 | OK. For task Lighting in Kitchen. Actual lengths to be confirmed as per Site., OLD QTY 62 MTR |  |  |
|  |  |  | Dimension: 17 X 14mm |  |  | Mtrs. |  |  |  |
|  |  |  | Accessory: Channel with Diffuse Cover |  |  |  |  |  |  |
|  |  |  | Lamp: 10W/Mt LED, 3000K, CRI=90+ |  |  |  |  |  |  |
|  |  |  | Lumen Output :100lm/Mt |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 10W/Mt |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  |  |  |  | 80W, 24V Dali Driver | 14.00 |  | 4992 | 69888 |
|  |  | 5 | LED Strip Light with Channel & Dotless Diffuse Cover |  | ENDO LIGHTING |  |  | 1251 | 160754 |
|  |  |  | Housing: Extruded aluminium Channel & Cover | COV4 | TSUKI-S-1714-5W/Mtr (IP65) | 128.50 | IP65 Required Actual Length to be confirmed as per site., OLD QTY 125 MTR & IP20 |  |  |
|  |  |  | Dimension: 17mm X 14mm |  |  | Mtrs. |  |  |  |
|  |  |  | Accessory: Channel with Diffuse Cover |  |  |  |  |  |  |
|  |  |  | Lamp: 5W/Mt LED, 3000K, CRI=90+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : 500lm/Mt |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 5W |  |  |  |  |  |  |
|  |  |  | IP65 |  |  |  |  |  |  |
|  |  |  |  |  | 80W, 24V Dali Driver | 14.00 |  | 4992 | 69888 |
|  |  | 6 | Flex Light with Top View |  | ENDO LIGHTING |  |  | 4425 | 40710 |
|  |  |  | Housing: Silicone | FLX1 | TSUKI-NF-DB-16X16-NS-130- 9W-IP67 | 9.20 | 14W/Mt is too high, (5W/Mt to 7W/Mt LED required) Actual Length to be confirmed as per site., OLD QTY 3.15 MTR |  |  |
|  |  |  | Dimension: 16 X 16mm |  |  | Mtrs. |  |  |  |
|  |  |  | Accessory: Fixing Clips |  |  |  |  |  |  |
|  |  |  | Lamp: 9W/Mt LED, 120° 3000K, CRI=85+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : 710lm/Mt |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 9W/Mt |  |  |  |  |  |  |
|  |  |  | IP67 |  |  |  |  |  |  |
|  |  |  |  |  | 80W, 24V Dali Driver | 2.00 |  | 4992 | 9984 |
|  |  |  |  |  |  |  | OLD QTY 32 NOS | 9045 | 298485 |
|  |  | 7 | Ceiling Recessed LED Down Light (IP Rated) |  | ENDO LIGHTING |  |  |  |  |
|  |  |  | Housing: Die cast Aluminium | IPR1 | GERD8901W-T | 33.00 |  |  |  |
|  |  |  | Dimension:Trim∅=60mm, Ht=95mm | White Trim |  | Nos |  |  |  |
|  |  |  | Lamp: 10.2W LED, 23° Fixed,3000K, CRI=90+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output : 446lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 10.2W |  |  |  |  |  |  |
|  |  |  | IP44 |  |  |  |  |  |  |
|  |  | 8 | In-ground LED Up lighter. |  |  |  |  | 9317 | 139755 |
|  |  |  |  | IUL1 | ELUIR1-70-0630-KN | 15.00 | Share the data sheet of this. |  |  |
|  |  |  | ELUIR1-70-0630-KN (Tsuki ingorund 5.8W, 9Deg, 3000K, 80 CRI, 70mm Dia, Non Dimmable ) |  |  | Nos |  |  |  |
|  |  | 9 | Profile System , Recessed Magnetic Track. |  |  |  |  | 5842 | 70104 |
|  |  |  | XR-CMT410 (48V DC Recess track with all accesoried) | PSY1 | Track | 12.00 | A recessed track with trim preferred. Share the data sheet of this. Drawing to be shared with Vendor to finalize unit modules & connectors. |  |  |
|  |  |  |  |  |  | Nos |  |  |  |
|  |  | 9a | Recessed Linear LED light in Profile System. |  | ENDO LIGHTING |  |  | 12491 | 112419 |
|  |  |  | Housing: Aluminum + PC | M1 | FL-24G | 9.00 | Quantity will change since length is lesser. Refer the shared render given by I.D. Share the IES file to evaluate. |  |  |
|  |  |  | Dimension: L=485mm W=25mm, D=46mm |  |  | Nos |  |  |  |
|  |  |  | Accessory: Recessing Track - Profile |  |  |  |  |  |  |
|  |  |  | Lamp: 24W/485mm LED, 3000K, CRI=90+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output : 1000lm/300mm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption:24W/485mm |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  | 9b | Recessed LED Twin Spot light in Profile System. |  | ENDO LIGHTING |  |  | 14514 | 174168 |
|  |  |  | Housing: Aluminum | M2 | GERS7013BA-T(Mini 40 spot) | 12.00 | Quantity will change. Refer the shared render given by I.D. Share the IES file to evaluate. |  |  |
|  |  |  | Dimension: TrimØ=45mm, L=94mm & Ht=52mm |  |  | Nos |  |  |  |
|  |  |  | Accessory: Recessing Track - Profile |  |  |  |  |  |  |
|  |  |  | Lamp: 10.2W LED 33°, 3000K, CRI=83+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output : 767lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 10.2W |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  | 10 | Ceiling Recessed LED Pin Spot Down Light. |  | GERD9898W-T |  |  | 5686 | 28430 |
|  |  |  | Housing: Die cast aluminium | PNS1 |  | 5.00 | The alternate is missing in received BOQ, NEW ITEM |  |  |
|  |  |  | Dimension: Trim∅=37mm, CutOut∅=35mm Ht=42mm. |  |  | Nos |  |  |  |
|  |  |  | Lamp: 3.1W LED 29°, Focusable, 3000K, CRI=93+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output: 178lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Cosumption: 3.1W |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  | 11 | Ceiling Recessed LED Pin Spot Down Light. |  |  |  |  | 10552 | 84416 |
|  |  |  | Housing: Die cast aluminium | PNS2 | GERD9869W-T | 8.00 | The alternate is missing in received BOQ, NEW ITEM |  |  |
|  |  |  | Dimension: Trim∅=60mm,Cut Out∅=50mm Ht=101mm. |  |  | Nos |  |  |  |
|  |  |  | Lamp: 10.3W LED 21°, Focusable, 3000K, CRI=93+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output: 446lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Cosumption: 10.3W |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  | 12 | Ceiling Recessed LED Down Light. |  | ENDO LIGHTING |  |  | 9621 | 355977 |
|  |  |  | Housing: Aluminium | RDN1 | GSTDA1800W-AMH30 | 37.00 | Kindly Suggest ACCORD (2000 TYPE - 19.6W Middle Beam Angle), OLD QTY 32 NOS |  |  |
|  |  |  | Dimension:∅=65mm, H=129mm. |  |  | Nos |  |  |  |
|  |  |  | Lamp: 19.8W LED 23°, Focusable, 3000K, CRI=93+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output: 1243lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Cosumption: 19.8W |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  | 13 | Ceiling Reccesed LED Down Light. |  | ENDO LIGHTING |  |  | 9621 | 221283 |
|  |  |  | Housing: Aluminium | RDN2 | GSTDA1800W-AWH30 | 23.00 | Kindly Suggest ACCORD (2000 TYPE - 19.6W Wide Beam Angle), OLD QTY 32 NOS |  |  |
|  |  |  | Dimension:L=∅=65mm, H=129m. |  |  | Nos |  |  |  |
|  |  |  | Lamp: 19.8W LED 33°, Focusable, 3000K, CRI=93+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output: 1368lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Cosumption: 19.8W |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  | 14 | Ceiling Reccesed LED Twin Down Light. (Black Trim Required) |  |  |  |  | 17792 | 569344 |
|  |  |  | Housing: Die cast aluminium ( Cust to Black Trim ) | RDN3 | GERD8849W-T | 32.00 | Black Trim Finish (I.D to confirm) This is OK. |  |  |
|  |  |  | Dimension: o 122 X65mm & H=115mm |  |  | Nos |  |  |  |
|  |  |  | Lamp:14.2W LED, 26°, Focusable, 3000K, CRI=93+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output : 1254lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 14.2W/Unit |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  | 15 | Ceiling Recessed LED Down Light |  |  |  |  | 11253 | 168795 |
|  |  |  | Housing: Die cast aluminium ( Cust to Black Trim ) | RDN4b | GERD8127WA-T (CUSTOMISED TO BLACK TRIM). | 15.00 | Black Trim Finish (In Bedroom 04 sloping ceiling) This is OK. |  |  |
|  |  |  | Dimension: ∅=108mm, Depth=100mm. |  |  | Nos |  |  |  |
|  |  |  | Lamp: 19.8W LED,19°, Focusable ,3000K CRI=96+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output : 1377lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 19.8W |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  | 16 | Ceiling Recessed LED Down Light |  |  |  |  | 11253 | 135036 |
|  |  |  | Ceiling Recessed LED Down Light | RDN4w | GERD8127WA-T | 12.00 | . NEW ITEM |  |  |
|  |  |  | Housing: Die cast aluminium |  |  | Nos |  |  |  |
|  |  |  | Dimension: ∅=108mm, Depth=100mm. |  |  |  |  |  |  |
|  |  |  | Lamp: 19.8W LED,19°, Focusable ,3000K CRI=96+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output : 1377lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 19.8W |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  | 17 | Ceiling Recessed LED Down Light |  |  |  |  | 13401 | 67005 |
|  |  |  | Housing: Die cast aluminium | RDN5 | GERD8202WB-T. | 5.00 | This is OK. |  |  |
|  |  |  | Dimension: ∅=108mm, Depth=133mm. |  |  | Nos |  |  |  |
|  |  |  | Lamp: 28.5W LED, 32°, Fixed Optics, 3000K CRI=83+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output : 1597lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 28.5W |  |  |  |  |  |  |
|  |  |  | IP20 |  |  |  |  |  |  |
|  |  | 18 | Surface Mounted LED Down Light. |  |  |  |  | 3908 | 54712 |
|  |  |  | Housing: Die cast aluminium | SDN1 | Abby Surface Down light 14W, 3000K. | 14.00 | OLD QTY 10 NOS. WE HAVE PROPOSED ALTERNATE ECONOMICAL OPTION IN Abby BRAND. |  |  |
|  |  |  | Dimension: ∅=152mm & H=38mm. |  |  | Nos |  |  |  |
|  |  |  | Lamp:15W LED, Flood Beam, Fixed, 3000K, CRI=85+(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output : 1209lm |  |  |  |  |  |  |
|  |  |  | Control Gear: LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 15W |  |  |  |  |  |  |
|  |  |  | IP44 Required |  |  |  |  |  |  |
|  |  | 19 | Ceiling Recessed LED Light, Fixed. | SDN2 | GERD8604W-T | 12.00 |  | 10345 | 124140 |
|  |  |  | Housing: Die cast aluminium |  |  | Nos |  |  |  |
|  |  |  | Dimension: ∅=108mm, Cutout∅=100 & Height=110mm. |  |  |  |  |  |  |
|  |  |  | Lamp:19.3W LED, 31° Fixed, 3000K, CRI=85(R9≥40+) |  |  |  |  |  |  |
|  |  |  | Lumen Output : 2014lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 19.3W |  |  |  |  |  |  |
|  |  |  | IP44 Required |  |  |  |  |  |  |
|  |  | 20 | Spike Mounted LED Projector Light. |  | GERS6922H-T | 11.00 | OLD QTY 6 | 10819 | 119009 |
|  |  |  | Housing: Alluminium | SPK1 |  | Nos |  |  |  |
|  |  |  | Dimension: ∅=61mm, Ht=141mm |  |  |  |  |  |  |
|  |  |  | Lamp:10W LED,23º, 3000K CRI=80+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : 758lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Non-Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 10.1W |  |  |  |  |  |  |
|  |  |  | IP65 |  |  |  | OLD QTY 6 | 7530 | 82830 |
|  |  | 21 | Spike Mounted LED Projector Light. |  | GERS7306H-T +GRB898N | 11.00 |  |  |  |
|  |  |  | Housing: Alluminium | SPK2 |  | nos |  |  |  |
|  |  |  | Dimension: ∅=35mm, Ht=63mm |  |  |  |  |  |  |
|  |  |  | Lamp:3W LED,45º, 3000K CRI=80+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : 230lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Non-Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 10.1W |  |  |  |  |  |  |
|  |  |  | IP65 |  |  |  |  |  |  |
|  |  | 22 | Underwater LED Up Lighter. |  |  |  |  |  |  |
|  |  |  | Housing: Stainless Steel | UWL3 |  |  | Regret |  |  |
|  |  |  | Dimension: TrimØ=81mm, Cut OutØ=79mm & Ht=100mm. |  |  |  |  |  |  |
|  |  |  | Lamp: 6W LED, 10°, 3000K, CRI=80+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : 576lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 6W |  |  |  |  |  |  |
|  |  |  | IP68 |  |  |  |  |  |  |
|  |  | 23 | Wall Mounted LED Light. |  | EWLR2-300-1030-K | 5.00 | OLD QTY 6 | 14811 | 74055 |
|  |  |  | Housing: Die cast Aluminium | WML1 |  | Nos |  |  |  |
|  |  |  | Dimension: ∅=300mm, Ht=57mm |  |  |  |  |  |  |
|  |  |  | Lamp:9W LED,92°×75°, 3000K CRI=80+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : 836lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Non-Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 9W |  |  |  |  |  |  |
|  |  |  | IP65 |  |  |  |  |  |  |
|  |  | 24 | Wall Recessed LED Light. |  |  |  | OLD QTY 10 | 7378 | 81158 |
|  |  |  | Housing: Die cast Aluminium | WRL1 | ELFR5-255-0630-K | 11.00 |  |  |  |
|  |  |  | Dimension: ∅=255mm, Ht=67mm |  |  | Nos |  |  |  |
|  |  |  | Lamp: 5.7W LED, 3000K CRI=80+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : CW=113 WW=104 |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Non-Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 5.7W |  |  |  |  |  |  |
|  |  |  | IP65 |  |  |  |  |  |  |
|  |  | 25 | Wall Recessed LED FootLight | WRL2 | ELFR2-72-0330-K | 21.00 |  | 5590 | 117390 |
|  |  |  | Housing: Die cast Aluminium |  |  | Nos |  |  |  |
|  |  |  | Dimension: ∅=72mm, Ht=75mm |  |  |  |  |  |  |
|  |  |  | Lamp: 3.1W LED, 3000K CRI=80+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : CW=25 WW=22 |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Non-Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 3.1W |  |  |  |  |  |  |
|  |  |  | IP65 |  |  |  |  |  |  |
|  |  | 26 | Wall Recessed LED Light. | WRL3 | ELFR5-255-0630-K | 6.00 | NEW ITEM | 7378 | 44268 |
|  |  |  | Housing: Die cast Aluminium |  |  | Nos |  |  |  |
|  |  |  | Dimension: ∅=255mm, Ht=67mm |  |  |  |  |  |  |
|  |  |  | Lamp: 5.7W LED, 3000K CRI=80+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : CW=113 WW=104 |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Non-Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 5.7W |  |  |  |  |  |  |
|  |  |  | IP65 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Total | 4,000,000 |
|  |  |  |  |  |  |  |  | GST 18% | 720,000 |
|  |  |  |  |  |  |  |  | Grand Total | 4,720,000 |

Underwater:

| PROJECT : RIVER GLADE VILLAS |  | BILL OF QUANTITIES FOR RIVER GLADE VILLAS - Villa 03 |  |  | Date: 24 SEPT 2024 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Notes : Included are luminaires suggested for - Interior Lighting for VILLA-03 Kindly Refer the detail specifications with this BOQ. If you Find any descripancies , please get it to our notice before placing final order. |  |  |  |  |  |  |  |  |
| Sr No | Specification Description |  | Symbol | Manufacturer |  | Total Quantity | Remarks |  |
| 22 | Underwater LED Up Lighter. |  |  | PUK Lighting |  |  |  |  |
|  | Housing: Stainless Steel |  | UWL3 | 301004 |  | 2.00 | Black Finish Required |  |
|  | Dimension: TrimØ=81mm, Cut OutØ=79mm & Ht=100mm. |  |  |  |  | Nos |  |  |
|  | Lamp: 6W LED, 10°, 3000K, CRI=80+ |  |  |  |  |  |  |  |
|  | Lumen Output : 576lm |  |  |  |  |  |  |  |
|  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |  |
|  | Power Consumption: 6W |  |  |  |  |  |  |  |
|  | IP68 |  |  |  |  |  |  |  |

| DATE: 24 OCT 2024 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ENDO ALTERNATES |  |  |  |  |  |  |  |
|  |  | Sr No | Specification Description | Symbol | Manufacturer | Total Quantity | Remarks/ L@W Comments | BEST PRICE | Total |
|  |  | 22 | Underwater LED Up Lighter. |  | PUK Lighting |  |  | 35890 | 71780 |
|  |  |  | Housing: Stainless Steel | UWL3 | 301004 | 2.00 | Same brand quoted which is proposed by consultant |  |  |
|  |  |  | Dimension: TrimØ=81mm, Cut OutØ=79mm & Ht=100mm. |  |  | nos |  |  |  |
|  |  |  | Lamp: 6W LED, 10°, 3000K, CRI=80+ |  |  |  |  |  |  |
|  |  |  | Lumen Output : 576lm |  |  |  |  |  |  |
|  |  |  | Control Gear: Remote LED Driver. Dali Dimmable. |  |  |  |  |  |  |
|  |  |  | Power Consumption: 6W |  |  |  |  |  |  |
|  |  |  | IP68 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Total | 71,780 |
|  |  |  |  |  |  |  |  | GST 18% | 12,920 |
|  |  |  |  |  |  |  |  | Grand Total | 84,700 |

Client scope:

Scaffolding wherever required to be provided.

Electrification wherever necessary to be provided.

Fabrication like poles etc. to be provided.

Fabricated poles if required to be provided.

Civil work if required.

Payment Terms

100% Advance

Company’s Bank Details

Bank Name: Central Bank of India

Account Number: 3167002097

Branch & IFS Code: Panaji & CBIN0280713

Terms & conditions apply.

Quotation valid for 10 days only.

If payment not received on time interest @ 15% will be levied per annum.

Any extra item if required to be charged as actual.

Warranty as per manufacturer not valid in case of surges, fluctuation, lightening etc.to be claimed directly with the

manufacturer without involving JP TECHATRONICS.

Solution done as per recommendation and request by client hence, JP TECHATRONICS or its employees should not be held responsible for any kind of malfunctioning of the system for any reasons.

Services like data backup, searching footage, port forwarding etc. to be charged as actuals.

Service calls post installation to be charged as actuals.